APB Verification Architecture in SystemVerilog

APB Overview

The Advanced Peripheral Bus (APB) is part of the AMBA protocol family developed by ARM. APB is optimized for low-power and low-bandwidth peripheral communication. It provides a simple interface with reduced complexity and is mainly used to connect peripherals such as UART, GPIO, Timers, and RTC modules.

The APB protocol operates using a simple state-based transfer mechanism consisting of IDLE, SETUP, and ACCESS phases. It supports read and write transactions with minimal control logic.

Chapter 1: APB Features

The key features of the APB Protocol are:

- Simple non-pipelined protocol

- Low power peripheral bus

- Separate read and write transfers

- PSTRB based byte-level write masking

- Single master and single/multiple slave support

- Two-phase transfer mechanism

- Address, control, and data phases

- PENABLE based access control

- PREADY based wait-state support

- PSLVERR based error indication

- Synchronous protocol operation

- Easy integration with low-speed peripherals

Chapter 2: APB Verification Plan

Verification involves studying the APB protocol specification, extracting protocol features to be tested, developing a verification strategy, implementing a verification environment, creating testcases, and achieving functional and code coverage closure.

2.1 Feature Extraction

Feature extraction involves identifying protocol features and behaviors that must be verified.

The following APB features are verified:

- Reset behavior

- Idle state verification

- Write transfer verification

- Write byte strobe (PSTRB) verification

- Read transfer verification

- Back-to-back transfers

- Wait-state handling using PREADY

- Error handling using PSLVERR

- Address stability during transfer

- Control signal validation

- PENABLE assertion timing

- PSEL assertion behavior

- Protocol sequencing

- Invalid transaction handling

- Simultaneous read/write prevention

2.2 Coverage Plan

Functional coverage is developed to ensure complete verification of APB protocol scenarios.

Coverage includes:

- Read and write operations

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Transfer type coverage

- Wait-state coverage

- Error response coverage

- Back-to-back transaction coverage

- Reset transition coverage

- Cross coverage between operation type and response signals

2.3 Checker Plan

The checker plan ensures APB protocol compliance.

Checkers are implemented for:

- Correct APB state transitions

- PENABLE assertion only during ACCESS phase

- PSEL stability during transfer

- Address and control stability during ACCESS phase

- Proper write and read sequencing

- PSTRB valid masking check

- Valid PREADY handling

- PSLVERR response checking

- Read data verification

- Reset signal behavior

2.4 Verification Environment Development

The APB verification environment is developed using SystemVerilog based layered testbench architecture.

The environment includes:

- Transaction class

- Generator

- Driver

- Monitor

- Reference Model

- Scoreboard

- Functional Coverage

- Environment class

- Test class

- Interface

- Top module

Chapter 3: APB Verification Environment Development

3.1 APB Verification Architecture

The APB verification environment follows a layered architecture where transactions are generated at high abstraction level and converted into pin-level activity.

Verification flow:

- Generator creates randomized or directed APB transactions.

- Driver converts transactions into APB bus activity.

- DUT processes APB transactions.

- Monitor samples APB interface signals.

- Reference model predicts expected outputs.

- Scoreboard compares expected and actual outputs.

- Coverage collector gathers functional coverage.

3.2 APB Verification Architecture Diagram

TB_TOP

TEST

ENV

GENERATOR

Ref_

Model

Score

board

Monitor

Driver

Interface

DUV

Chapter 4: APB Verification Components

4.1 Transaction Class

The transaction class represents APB transfer information in transaction-level format.

It contains:

- APB address

- Write data

- Read data

- Transfer type

- PWRITE signal

- PSTRB signal

- PSEL signal

- PENABLE signal

- PREADY signal

- PSLVERR signal

Responsibilities

- Randomization of APB transactions

- Constraint generation

- Protocol field storage

- Communication between components

Example Fields

class apb_transaction;

rand bit [31:0] paddr;

rand bit [31:0] pwdata;

bit [31:0] prdata;

rand bit [ 3:0] pstrb;

rand bit        pwrite;

bit        psel;

bit        penable;

bit        pready;

bit        pslverr;

endclass

4.2 Generator

The generator creates APB stimulus transactions.

It generates:

- Directed transactions

- Random transactions

- Error scenarios

- Wait-state scenarios

- Back-to-back transactions

Responsibilities

- Random transaction generation

- Constraint control

- Sending transactions to driver using mailbox

- Controlling transaction count

Generator Flow

Generator -> Mailbox -> Driver

4.3 Driver

The driver converts transaction-level activity into APB pin-level protocol activity.

The driver receives transactions from the generator and drives APB signals through the interface.

Responsibilities

- Driving APB signals

- Implementing APB SETUP and ACCESS phases

- Applying reset

- Driving read and write operations

- Handling wait states

- Synchronizing with clock

APB Write Transfer Sequence

- Assert PSEL

- Drive address and write data

- Assert PWRITE

- Assert PENABLE in next cycle

- Wait for PREADY

- Complete transfer

APB Read Transfer Sequence

- Assert PSEL

- Drive address

- Deassert PWRITE

- Assert PENABLE

- Wait for PREADY

- Sample PRDATA

4.4 Monitor

The monitor observes APB bus activity and converts signal-level information into transaction-level data.

The monitor is passive and does not drive any signals.

Responsibilities

- Sampling APB interface signals

- Collecting read and write transactions

- Detecting protocol activity

- Sending transactions to scoreboard and coverage collector

Monitor Checks

- PSEL assertion

- PENABLE timing

- Address stability

- Wait-state handling

- Read data capture

4.5 Reference Model

The reference model predicts expected APB behavior.

It acts as a golden model and maintains expected data for comparison.

Responsibilities

- Predicting expected read data

- Maintaining memory model

- Handling write updates

- Generating expected transaction results

Reference Model Operation

- During write transfer:

  - Store data at given address

- During read transfer:

  - Retrieve expected data from stored address

4.6 Scoreboard

The scoreboard compares DUT outputs with reference model outputs.

It validates protocol correctness and data integrity.

Responsibilities

- Comparing expected and actual transactions

- Logging PASS/FAIL messages

- Reporting mismatches

- Tracking verification status

Comparison Checks

- Address match

- Read data match

- Transfer type match

- Error response validation

- Wait-state correctness

Example Scoreboard Message

WRITE PASS : Address = 0x10 Data = 0x55

READ PASS  : Address = 0x10 Data = 0x55

4.7 Functional Coverage

Functional coverage measures verification completeness.

Coverage Points

- Read operation coverage

- Write operation coverage

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Wait-state coverage

- PSLVERR coverage

- Back-to-back transfer coverage

- Reset coverage

Cross Coverage

- Read/Write vs Address

- Read/Write vs Wait-state

- Transfer type vs Error response

4.8 Environment Class

The environment class instantiates and connects all verification components.

Components instantiated inside environment

- Generator

- Driver

- Monitor

- Reference model

- Scoreboard

- Coverage collector

Responsibilities

- Connecting mailboxes

- Connecting virtual interfaces

- Starting component execution

- Managing simulation flow

4.9 Test Class

The test class controls simulation scenarios.

Responsibilities

- Creating environment

- Configuring testcases

- Starting random or directed tests

- Applying reset sequence

- Controlling simulation duration

Example Testcases

- Sanity test

- Write only test

- Read only test

- Random transfer test

- Wait-state test

- Error response test

- Back-to-back transfer test

4.10 Interface

The APB interface groups APB signals together and simplifies DUT connectivity.

APB Interface Signals

interface apb_if(input logic PCLK);

logic PRESETn;

logic PSEL;

logic PENABLE;

logic PWRITE;

logic [31:0] PADDR;

logic [31:0] PWDATA;

logic [31:0] PRDATA;

logic [ 3:0] PSTRB;

logic PREADY;

logic PSLVERR;

endinterface

4.11 Why Virtual Interface?

The interface is a static construct, whereas verification components such as driver and monitor are dynamic class-based objects.

Since interfaces cannot be directly instantiated inside classes, virtual interfaces are used.

Definition

A virtual interface is a pointer to an actual interface instance.

virtual apb_if vif;

Advantages

- Simplifies DUT connection

- Allows class-based access to interface signals

- Improves reusability

- Enables protocol abstraction

Chapter 5: APB Testcase Flow

Sanity Test Steps

- Apply reset

- Perform write operation to address 0x10 with data 0x55

- Wait for one clock cycle

- Perform read operation from address 0x10

- Compare read data with expected value

- Report PASS or FAIL

Chapter 6: Running Simulation

Compilation

make sanity

make regress

Simulation

Chapter 7: Closure Report

Verification closure is achieved after:

- All directed testcases pass

- Random testcases achieve stable results

- Functional coverage reaches target percentage

- Code coverage goals are achieved

- No protocol violations remain

- Scoreboard mismatches are resolved

References

The APB verification architecture has been developed by referring to the RAM verification architecture document and layered verification methodology structure. fileciteturn0file3

# APB Verification Architecture in SystemVerilog

## APB Overview

The Advanced Peripheral Bus (APB) is part of the AMBA protocol family developed by ARM. APB is optimized for low-power and low-bandwidth peripheral communication. It provides a simple interface with reduced complexity and is mainly used to connect peripherals such as UART, GPIO, Timers, and RTC modules.

The APB protocol operates using a simple state-based transfer mechanism consisting of IDLE, SETUP, and ACCESS phases. It supports read and write transactions with minimal control logic.

# Chapter 1: APB Features

The key features of the APB Protocol are:

- Simple non-pipelined protocol

- Low power peripheral bus

- Separate read and write transfers

- PSTRB based byte-level write masking

- Single master and single/multiple slave support

- Two-phase transfer mechanism

- Address, control, and data phases

- PENABLE based access control

- PREADY based wait-state support

- PSLVERR based error indication

- Synchronous protocol operation

- Easy integration with low-speed peripherals

# Chapter 2: APB Verification Plan

Verification involves studying the APB protocol specification, extracting protocol features to be tested, developing a verification strategy, implementing a verification environment, creating testcases, and achieving functional and code coverage closure.

## 2.1 Feature Extraction

Feature extraction involves identifying protocol features and behaviors that must be verified.

The following APB features are verified:

- Reset behavior

- Idle state verification

- Write transfer verification

- Write byte strobe (PSTRB) verification

- Read transfer verification

- Back-to-back transfers

- Wait-state handling using PREADY

- Error handling using PSLVERR

- Address stability during transfer

- Control signal validation

- PENABLE assertion timing

- PSEL assertion behavior

- Protocol sequencing

- Invalid transaction handling

- Simultaneous read/write prevention

## 2.2 Coverage Plan

Functional coverage is developed to ensure complete verification of APB protocol scenarios.

Coverage includes:

- Read and write operations

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Transfer type coverage

- Wait-state coverage

- Error response coverage

- Back-to-back transaction coverage

- Reset transition coverage

- Cross coverage between operation type and response signals

## 2.3 Checker Plan

The checker plan ensures APB protocol compliance.

Checkers are implemented for:

- Correct APB state transitions

- PENABLE assertion only during ACCESS phase

- PSEL stability during transfer

- Address and control stability during ACCESS phase

- Proper write and read sequencing

- PSTRB valid masking check

- Valid PREADY handling

- PSLVERR response checking

- Read data verification

- Reset signal behavior

## 2.4 Verification Environment Development

The APB verification environment is developed using SystemVerilog based layered testbench architecture.

The environment includes:

- Transaction class

- Generator

- Driver

- Monitor

- Reference Model

- Scoreboard

- Functional Coverage

- Environment class

- Test class

- Interface

- Top module

# Chapter 3: APB Verification Environment Development

## 3.1 APB Verification Architecture

The APB verification environment follows a layered architecture where transactions are generated at high abstraction level and converted into pin-level activity.

Verification flow:

- Generator creates randomized or directed APB transactions.

- Driver converts transactions into APB bus activity.

- DUT processes APB transactions.

- Monitor samples APB interface signals.

- Reference model predicts expected outputs.

- Scoreboard compares expected and actual outputs.

- Coverage collector gathers functional coverage.

## 3.2 APB Verification Architecture Diagram

TB_TOP

TEST

ENV

GENERATOR

Ref_

Model

Score

board

Monitor

Driver

Interface

DUV

(A Class Based Layered TB)

# Chapter 4: APB Verification Components

## 4.1 Transaction Class

The transaction class represents APB transfer information in transaction-level format.

It contains:

- APB address

- Write data

- Read data

- Transfer type

- PWRITE signal

- PSTRB signal

- PSEL signal

- PENABLE signal

- PREADY signal

- PSLVERR signal

### Responsibilities

- Randomization of APB transactions

- Constraint generation

- Protocol field storage

- Communication between components

### Example Fields

class apb_transaction;

rand bit [31:0] paddr;

rand bit [31:0] pwdata;

bit [31:0] prdata;

rand bit [ 3:0] pstrb;

rand bit        pwrite;

bit        psel;

bit        penable;

bit        pready;

bit        pslverr;

endclass

## 4.2 Generator

The generator creates APB stimulus transactions.

It generates:

- Directed transactions

- Random transactions

- Error scenarios

- Wait-state scenarios

- Back-to-back transactions

### Responsibilities

- Random transaction generation

- Constraint control

- Sending transactions to driver using mailbox

- Controlling transaction count

### Generator Flow

Generator -> Mailbox -> Driver

## 4.3 Driver

The driver converts transaction-level activity into APB pin-level protocol activity.

The driver receives transactions from the generator and drives APB signals through the interface.

### Responsibilities

- Driving APB signals

- Implementing APB SETUP and ACCESS phases

- Applying reset

- Driving read and write operations

- Handling wait states

- Synchronizing with clock

### APB Write Transfer Sequence

- Assert PSEL

- Drive address and write data

- Assert PWRITE

- Assert PENABLE in next cycle

- Wait for PREADY

- Complete transfer

### APB Read Transfer Sequence

- Assert PSEL

- Drive address

- Deassert PWRITE

- Assert PENABLE

- Wait for PREADY

- Sample PRDATA

## 4.4 Monitor

The monitor observes APB bus activity and converts signal-level information into transaction-level data.

The monitor is passive and does not drive any signals.

### Responsibilities

- Sampling APB interface signals

- Collecting read and write transactions

- Detecting protocol activity

- Sending transactions to scoreboard and coverage collector

### Monitor Checks

- PSEL assertion

- PENABLE timing

- Address stability

- Wait-state handling

- Read data capture

## 4.5 Reference Model

The reference model predicts expected APB behavior.

It acts as a golden model and maintains expected data for comparison.

### Responsibilities

- Predicting expected read data

- Maintaining memory model

- Handling write updates

- Generating expected transaction results

### Reference Model Operation

- During write transfer:

  - Store data at given address

- During read transfer:

  - Retrieve expected data from stored address

## 4.6 Scoreboard

The scoreboard compares DUT outputs with reference model outputs.

It validates protocol correctness and data integrity.

### Responsibilities

- Comparing expected and actual transactions

- Logging PASS/FAIL messages

- Reporting mismatches

- Tracking verification status

### Comparison Checks

- Address match

- Read data match

- Transfer type match

- Error response validation

- Wait-state correctness

### Example Scoreboard Message

WRITE PASS : Address = 0x10 Data = 0x55

READ PASS  : Address = 0x10 Data = 0x55

## 4.7 Functional Coverage

Functional coverage measures verification completeness.

### Coverage Points

- Read operation coverage

- Write operation coverage

- Address coverage

- Data coverage

- PSTRB byte-enable coverage

- Wait-state coverage

- PSLVERR coverage

- Back-to-back transfer coverage

- Reset coverage

### Cross Coverage

- Read/Write vs Address

- Read/Write vs Wait-state

- Transfer type vs Error response

## 4.8 Environment Class

The environment class instantiates and connects all verification components.

### Components instantiated inside environment

- Generator

- Driver

- Monitor

- Reference model

- Scoreboard

- Coverage collector

### Responsibilities

- Connecting mailboxes

- Connecting virtual interfaces

- Starting component execution

- Managing simulation flow

## 4.9 Test Class

The test class controls simulation scenarios.

### Responsibilities

- Creating environment

- Configuring testcases

- Starting random or directed tests

- Applying reset sequence

- Controlling simulation duration

### Example Testcases

- Sanity test

- Write only test

- Read only test

- Random transfer test

- Wait-state test

- Error response test

- Back-to-back transfer test

## 4.10 Interface

The APB interface groups APB signals together and simplifies DUT connectivity.

### APB Interface Signals

interface apb_if(input logic PCLK);

logic PRESETn;

logic PSEL;

logic PENABLE;

logic PWRITE;

logic [31:0] PADDR;

logic [31:0] PWDATA;

logic [31:0] PRDATA;

logic [ 3:0] PSTRB;

logic PREADY;

logic PSLVERR;

endinterface

## 4.11 Why Virtual Interface?

The interface is a static construct, whereas verification components such as driver and monitor are dynamic class-based objects.

Since interfaces cannot be directly instantiated inside classes, virtual interfaces are used.

### Definition

A virtual interface is a pointer to an actual interface instance.

virtual apb_if vif;

### Advantages

- Simplifies DUT connection

- Allows class-based access to interface signals

- Improves reusability

- Enables protocol abstraction

# Chapter 5: APB Testcase Flow

## Sanity Test Steps

- Apply reset

- Perform write operation to address 0x10 with data 0x55

- Wait for one clock cycle

- Perform read operation from address 0x10

- Compare read data with expected value

- Report PASS or FAIL

# Chapter 6: Running Simulation

## Compilation

make sanity

make regress

## Simulation

# Chapter 7: Closure Report

Verification closure is achieved after:

- All directed testcases pass

- Random testcases achieve stable results

- Functional coverage reaches target percentage

- Code coverage goals are achieved

- No protocol violations remain

- Scoreboard mismatches are resolved

# References

The APB verification architecture has been developed by referring to the RAM verification architecture document and layered verification methodology structure. fileciteturn0file3